Metropolitan Washington Regional ITS Architecture
Validation Package Instructions

February 14, 2005 
Page 1 of 7

Instructions on How To Review the Validation Package
1. Staff has produced DRAFT “validation packages” for the regional ITS architecture. There are five packages available, which are a Virginia Package, Maryland Package, DC Package, Transit Package and Whole Package. Each package contains several Regional ITS Elements by geography except Transit Package, which includes primarily WMATA and other regional transit related elements. The Whole Package includes all elements in the region.  The validation packages are in PowerPoint format, but are a product of the “Turbo Architecture” software. 
2. The first cover page (example below) shows what elements in the package need to be reviewed.


[image: image14.emf]Washington Metropolitan Area Transit

Authority (WMATA)

WMATA Transit Bus (BTRA)

Local Signal Agencies

Local Field Equipment

local signal priority request

Existing

Planned


3. The second page shows the regional ITS architecture framework. It is also called “Sausage Diagram” in the National ITS Architecture.


[image: image2]
4. The following pages may be repeated depending on how many elements are in the package. Each initial page of this section shows the description of the element that needs to be reviewed. Please review the description of the element and provide your comments if applicable.

[image: image3]
5. The next page extends the “Sausage Diagram” by putting certain architecture elements into appropriate Architecture Entities, and also shows in which Architecture Entity Box   the target element sits by underlining the element with bold –Italic font. Due to the small font size required for the detailed chart, we recommend you use the “zoom” command on your PowerPoint or Word software to better view it on-screen if you have trouble reading a print-out.

[image: image4]
6. The next pages show the Interconnect Diagram of the target element. (The highlighted one is the element that needs to be reviewed). Please provide comments if necessary.

[image: image5]
7. The next page shows in which Architecture Entity Box the interconnected element(s) sit  in the “Sausage Diagram” by underlining the elements with bold –Italic font.

[image: image6]
8. Then follows pages show the “information diagram” for each interconnect pair elements. The highlighted one is the element that needs to be reviewed. The information flow definitions are in the Appendix C. The directional arrow of the information flow line indicates the source and destination .  Two arrows on the same information flow line means this particular information flow is a two-way flow.

[image: image7]
9. The following pages will repeat until the last interconnect element. Then the first Element in this package is done. 
10.  This procedure will repeat until the last element in the list.   

Local Field Equipment Interconnect Diagram








[image: image1][image: image8.emf]Local Signal Center

Local Field Equipment

To Be Reviewed

2 2005

Metropolitan Washington Area ITS Architecture 

ITS Elements

[image: image9.emf]Wide Area Wireless(Mobile)Communications

Wireline(Fixed-Point to Fixed-Point)Communications

Metropolitan Washington Area ITS Architecture  Framework

 Maintenance   Countruction

Vehicle

Maintenance Counstruction

Management

Archived Data

Management

Fleet and

Freight

Management

Transit

Management

Emissions

Management

Information

Service

Provider

Dedicated Short Range

Communications

Commercial

Vehicle

Check

Parking

Management

Toll

Collect

Roadway

Emergency

Vehicle

Vehicle to Vehicle Communications

Commercial

Vehicle

Transit

Vehicle

Vehicle

Commercial

Vehicle

Administration

Toll

Administration

Emergency

Management

Traffic Management

Roadside Vehicles

Centers

Personal

Information

Access

Remote Traveler Support

Travelers

[image: image10.emf]Element

Architecture

Description   

 Stakeholder

Element    

Local Signal Agencies

Local Field Equipment

These signal control devices are typically located along the roadside and are 

controlled by operators in the local signal centers.

[image: image11.emf]Metropolitan Washington Area ITS Architecture  - Local Field Equipment

Remote Traveler Support

Centers Emergency Management

Fleet and Freight Management

Emissions Management

Maintenance and Construction Management

Transit Management

Toll Administration

Traffic Management Archive Data Management Information Service Provider

Vehicle to Vehicle Communications

Wide Area Wireless(Mobile)Communications

Wireline(Fixed-Point to Fixed-Point)Communications

 Maintenance  Construction Vehicle

Dedicated Short Range

Communications

Roadway Parking Management Toll Collection Commercial Vehicle Check

Emergency

Vehicle

Commercial

Vehicle

Transit

Vehicle

Vehicle

Roadside Vehicles

Personal Information Access

Travelers

FEMA

CapCOM Center

WMATA Transit Police (MTPD) Operations 

Control Center

WMATA Transit Bus (BTRA)

Local Transit Vehicle

VDOT NOVA Safety Service Patrol

CAPWIN

Smart Tag Center

Greenway Center

Personal Traveler Information Devices 

WMATA In-Station Display System/Kiosk

MWAA Center

MDSHA CHART

Regional Transit Electronic Clearinghouse

WMATA Rail Operation and Bus Operations 

Control Center (ROCC-BOCC)

VRE Center

MWAA Center

Maryland State Transit Management Center

Local Transit Center

VDOT NOVA Maintenance and Construction 

Operations/Snow Operations

MDSHA CHART

DCDOT ITMS/TOC

Virginia State Police Center

VDOT NOVA STSS

VDOT NOVA Smart Traffic Center (STC)

VDOT NOVA Safety Service Patrol

VDOT NOVA CSC

National Park Services

MWAA Center

Mobile Unified Command Center

MEMA

MDSHA CHART

Local Signal Center

Local Public Safety Center

E-911 Communication Centers

DC Public Safety and Emergency Center

CAPWIN

CapCOM Center

VDOT Video Clearinghouse

VDOT NOVA STSS

VDOT NOVA Smart Traffic Center (STC)

National Park Services

Mobile Unified Command Center

MDSHA CHART

Local Signal Center

DCDOT ITMS/TOC

DC Public Safety and Emergency Center

VDOT Mobility Data Store

RITIS

CapCOM Center

WMATA Administration Support/WMATA 

Enterprise Geographic Information System 

(EGIS)

VDOT NOVA Smart Traffic Center (STC)

Smart Travel Lab

NVTC Center

MWCOG Database

MDSHA CHART

Local Signal Center

I-95 Corridor Coalition Information Exchange 

Network (IEN)

DCDOT ITMS/TOC

CAPWIN

Local Field Equipment

Regional Toll Road

WMATA Parking Management 

Center

MWAA Center

Maryland State Transit 

Management Center

DC Field Equipment

RITIS

CapCOM Center

WMATA Customer Services Center (CRCS)/Website 

Services/Radio

VDOT Video Clearinghouse

VDOT Public Affairs

VDOT NOVA Smart Traffic Center (STC)

VDOT NOVA CSC

Smart Travel Lab

Private Sector ISP Centers

MWAA Center

MDSHA CHART

Maryland State Transit Management Center

Local Transit Center

Local Signal Center

I-95 Corridor Coalition Information Exchange Network 

(IEN)

DCDOT ITMS/TOC

[image: image12.emf]Local Signal Agencies

Local Field Equipment

Local Signal Agencies

Local Signal Center

Local Transit Agencies

Local Transit Vehicle

VDOT NOVA District

VDOT NOVA Maintenance and

Construction Operations/Snow

Operations

VDOT NOVA District

VDOT NOVA Safety Service Patrol

Washington Metropolitan Area Transit

Authority (WMATA)

WMATA Transit Bus (BTRA)

Existing

Planned

[image: image13.emf]Metropolitan Washington Area ITS Architecture  - Local Field Equipment Interconnect Element

Remote Traveler Support

Centers Emergency Management

Fleet and Freight Management

Emissions Management

Maintenance and Construction Management

Transit Management

Toll Administration

Traffic Management Archive Data Management Information Service Provider

Vehicle to Vehicle Communications

Wide Area Wireless(Mobile)Communications

Wireline(Fixed-Point to Fixed-Point)Communications

 Maintenance  Construction Vehicle

Dedicated Short Range

Communications

Roadway Parking Management Toll Collection Commercial Vehicle Check

Emergency

Vehicle

Commercial

Vehicle

Transit

Vehicle

Vehicle

Roadside Vehicles

Personal Information Access

Travelers

FEMA

CapCOM Center

WMATA Transit Police (MTPD) Operations 

Control Center

WMATA Transit Bus (BTRA)

Local Transit Vehicle

VDOT NOVA Safety Service 

Patrol

CAPWIN

Smart Tag Center

Greenway Center

Personal Traveler Information Devices 

WMATA In-Station Display System/Kiosk

MWAA Center

MDSHA CHART

Regional Transit Electronic Clearinghouse

WMATA Rail Operation and Bus Operations 

Control Center (ROCC-BOCC)

VRE Center

MWAA Center

Maryland State Transit Management Center

Local Transit Center

VDOT NOVA Maintenance and Construction 

Operations/Snow Operations

MDSHA CHART

DCDOT ITMS/TOC

Virginia State Police Center

VDOT NOVA STSS

VDOT NOVA Smart Traffic Center (STC)

VDOT NOVA Safety Service Patrol

VDOT NOVA CSC

National Park Services

MWAA Center

Mobile Unified Command Center

MEMA

MDSHA CHART

Local Signal Center

Local Public Safety Center

E-911 Communication Centers

DC Public Safety and Emergency Center

CAPWIN

CapCOM Center

VDOT Video Clearinghouse

VDOT NOVA STSS

VDOT NOVA Smart Traffic Center (STC)

National Park Services

Mobile Unified Command Center

MDSHA CHART

Local Signal Center

DCDOT ITMS/TOC

DC Public Safety and Emergency Center

VDOT Mobility Data Store

RITIS

CapCOM Center

WMATA Administration Support/WMATA 

Enterprise Geographic Information System 

(EGIS)

VDOT NOVA Smart Traffic Center (STC)

Smart Travel Lab

NVTC Center

MWCOG Database

MDSHA CHART

Local Signal Center

I-95 Corridor Coalition Information Exchange 

Network (IEN)

DCDOT ITMS/TOC

CAPWIN

Local Field Equipment

Regional Toll Road

WMATA Parking Management 

Center

MWAA Center

Maryland State Transit 

Management Center

DC Field Equipment

RITIS

CapCOM Center

WMATA Customer Services Center (CRCS)/Website 

Services/Radio

VDOT Video Clearinghouse

VDOT Public Affairs

VDOT NOVA Smart Traffic Center (STC)

VDOT NOVA CSC

Smart Travel Lab

Private Sector ISP Centers

MWAA Center

MDSHA CHART

Maryland State Transit Management Center

Local Transit Center

Local Signal Center

I-95 Corridor Coalition Information Exchange Network 

(IEN)

DCDOT ITMS/TOC

